Summary

Against the backdrop of smartphone ubiquity, the unpredictability of battery depletion remains a critical challenge driven by stochastic user behaviors and complex environmental interactions. To accurately predict the state of charge (SOC) evolution and time-to-empty (TTE) under realistic conditions, we develop a Physically-Informed Coupled Electrochemical Framework. This continuous-time system integrates electrochemical dynamics with behavior-driven loads to quantify the diverse and interacting drivers of energy consumption.

Our methodology is grounded in a First-order RC Thévenin Equivalent Circuit Model (ECM), which captures transient polarization effects and voltage response dynamics from first principles. To bridge the gap between high-level human activity and physical power demand, we design a Behavior-Driven Load Generation (B2L) Interface that dynamically decomposes aggregate usage into distinct subsystem loads, including display, CPU, and network modules. These electrical models are bi-directionally coupled with a Lumped Thermal Dynamics Model, where current-induced Joule heating alters the battery's internal resistance, and ambient temperature fluctuations constrain the electrochemical reaction rates.

Quantitative simulations reveal precise insights into the mechanisms of battery drain. We derived the specific display power law with a unit-area coefficient of $\alpha=20.7 \text{ mW/cm}^2$ and a non-linear brightness exponent of $\gamma=1.8$. A critical "Penalty Gap" was identified in network usage: under heavy gaming loads, a degradation in signal strength from -60 dBm to -110 dBm precipitates a 40% reduction in TTE. Furthermore, the battery exhibits distinct Thermal Asymmetry: exposure to cold environments ($10^\circ\text{C}$) reduces TTE by 37%, whereas high temperatures ($40^\circ\text{C}$) cause only a 13% reduction, confirming that cold stress is the primary environmental threat compared to heat. Additionally, the constant-current to constant-voltage charging transition threshold was identified at $z_{knee} \approx 0.83$.

Validation against the NASA battery aging dataset using Leave-One-Out Cross-Validation (LOOCV) demonstrates robust predictive accuracy, with relative errors consistently maintained within 10%. Sensitivity analysis attributes the highest variance in TTE to signal strength parameters ($\approx 56\%$), significantly outweighing CPU load variations ($\approx 6\%$), establishing a dominant drain ranking of Radio Activity > Display Brightness > CPU Load. Based on these findings, we recommend that operating systems implement aggressive "suspend-to-RAM" policies in weak signal zones and that users employ Thermal pre-conditioning in cold environments to mitigate impedance-induced capacity loss.